CISCO NSO

Service Development Guide

VRF Creation Service (RFS) L3VPN Customer Facing Service (CFS) Stacked CFS → RFS Architecture

IOS-XR | Python | YANG | RESTCONF

May 2026

# 1. Overview & Architecture

This document covers two NSO service packages built and tested hands-on:

- vrf_creation_ios_xr — Resource Facing Service (RFS) that directly configures VRF on IOS-XR routers

- l3vpn-cfs — Customer Facing Service (CFS) that takes simple inputs and automatically calls the VRF RFS

## 1.1 The Stacked Service Concept

NSO services can be layered so that an engineer only types simple inputs and the system figures out all the technical details automatically.

Restaurant Analogy Customer (Engineer) -> says 'Burger Meal' (simple input) Waiter (CFS) -> takes order, tells kitchen what to make Chef (RFS) -> makes the actual food (pushes config to router) The customer NEVER talks to the kitchen directly. The CFS NEVER touches a router directly.

## 1.2 Architecture Diagram

Engineer / Portal / API ¦ ¦ l3vpn-cfs ZTP-VPN vpn-id 100 pe-device core-rtr01 ¦ (only 3 inputs needed) v ┌─────────────────────────┐ │ l3vpn-cfs (CFS) │ <-- Package 2 (NEW) │ The Waiter │ Takes simple input │ │ Calculates RT, VRF name └──────────┬──────────────┘ Calls the RFS ¦ ¦ creates instance of v ┌─────────────────────────┐ │ vrf_creation_ios_xr │ <-- Package 1 (YOUR EXISTING) │ (RFS) The Chef │ Receives calculated values │ │ Pushes config to router └──────────┬──────────────┘ ¦ v IOS-XR Router (VRF configured)

## 1.3 CFS vs RFS — Key Rules

| CFS (Customer Facing Service) | RFS (Resource Facing Service) |
| --- | --- |
| Takes simple inputs from engineer | Takes technical inputs from CFS |
| Auto-calculates RT, VRF names, etc. | Directly configures devices |
| Calls other services (RFS) | Has XML template or Python config push |
| NEVER touches a router directly | NEVER takes input from a human directly |
| Think: Waiter | Think: Chef |

# 2. Package 1 — VRF Creation Service (RFS)

This is the lower-layer service that directly configures VRF on IOS-XR routers. It was built first and becomes the RFS in the stacked architecture.

## 2.1 Package Creation Command

cd /nso/run/packages ncs-make-package --service-skeleton python vrf_creation_ios_xr

## 2.2 YANG Model

File location: packages/vrf_creation_ios_xr/src/yang/vrf_creation_ios_xr.yang

module vrf_creation_ios_xr { namespace "http://com/example/vrf_creation_ios_xr"; prefix vrf_creation_ios_xr; import ietf-inet-types { prefix inet; } import tailf-ncs { prefix ncs; } list vrf_creation_ios_xr { key name; uses ncs:service-data; ncs:servicepoint "vrf_creation_ios_xr"; leaf name { type string; } leaf-list device { type leafref { path "/ncs:devices/ncs:device/ncs:name"; } } leaf vrf { type string { length "1..32"; } } leaf import_rt { type string; mandatory true; } leaf export_rt { type string; mandatory true; } } }

### 2.2.1 YANG Model — Line by Line Explanation

| YANG Line | What It Does |
| --- | --- |
| module vrf_creation_ios_xr { } | Defines the module name — must match filename |
| namespace "http://..." | Unique identifier for this module globally |
| import tailf-ncs { prefix ncs; } | Import NSO library — gives access to service functions |
| list vrf_creation_ios_xr { } | A list where each entry = one service instance |
| key name; | The unique key for each service instance |
| uses ncs:service-data; | REQUIRED — gives the service rollback, re-deploy, check-sync |
| ncs:servicepoint "..."; | Links YANG model to Python callback code |
| leaf-list device { } | Accepts MULTIPLE device names (list of devices) |
| type leafref { path ... } | Validates that device exists in NSO device list |
| leaf vrf { length 1..32 } | VRF name — max 32 characters enforced by YANG |
| mandatory true; | Field is required — cannot be left empty |

## 2.3 XML Service Template

File location: packages/vrf_creation_ios_xr/templates/vrf_creation_ios_xr-template.xml

The XML template maps service model values to actual device configuration. Variables in {curly braces} are replaced with real values at runtime.

<config-template xmlns="http://tail-f.com/ns/config/1.0" servicepoint="vrf_creation_ios_xr"> <devices xmlns="http://tail-f.com/ns/ncs"> <device> <name>{/device}</name> <!-- loops over each device in leaf-list --> <config> <vrf xmlns="http://tail-f.com/ned/cisco-ios-xr"> <vrf-list> <name>{/vrf}</name> <!-- VRF name from service input --> <address-family> <ipv4> <unicast> <import> <route-target> <address-list> <name>{import_rt}</name> <!-- import RT value --> </address-list> </route-target> </import> <export> <route-target> <address-list> <name>{export_rt}</name> <!-- export RT value --> </address-list> </route-target> </export> </unicast> </ipv4> </address-family> </vrf-list> </vrf> </config> </device> </devices> </config-template>

### 2.3.1 XML Template — Line by Line Explanation

| XML Element | What It Does |
| --- | --- |
| {/device} | Loops over every device in the leaf-list and applies config to each |
| {/vrf} | Replaced with the VRF name typed by engineer |
| {import_rt} | Replaced with the import route-target value |
| {export_rt} | Replaced with the export route-target value |
| xmlns="http://tail-f.com/ned/cisco-ios-xr" | Tells NSO which NED to use (IOS-XR) |
| servicepoint="vrf_creation_ios_xr" | Links template to the correct service point |

## 2.4 How to Use the VRF RFS Service

### 2.4.1 NSO CLI

# Enter config mode ncs_cli -C -u admin config # Create service instance vrf_creation_ios_xr MY-VRF device [ core-rtr01 ] vrf BLUE-VRF import_rt 65000:100 export_rt 65000:100 # Check what will be pushed BEFORE committing commit dry-run outformat native # Commit and push to router commit

### 2.4.2 RESTCONF API

curl --location --request POST \ 'http://NSO_IP:8080/restconf/data/vrf_creation_ios_xr:vrf_creation_ios_xr' \ --header 'Content-Type: application/yang-data+json' \ --header 'Accept: application/yang-data+json' \ --user developer:C1sco12345 \ --data '{ "vrf_creation_ios_xr:vrf_creation_ios_xr": { "name": "MY-VRF", "device": ["core-rtr01"], "vrf": "BLUE-VRF", "import_rt": "65000:100", "export_rt": "65000:100" } }'

## 2.5 Compile and Load the Package

# Compile YANG model cd /nso/run/packages/vrf_creation_ios_xr/src make # Load into NSO ncs_cli -C -u admin packages reload # Expected output: reload-result { package vrf_creation_ios_xr result true }

# 3. Package 2 — L3VPN Customer Facing Service (CFS)

The CFS is the upper-layer service. The engineer only provides 3 simple inputs. The CFS automatically calculates all technical values and calls the VRF RFS to do the actual configuration.

Internal Mapping — The Key Concept Engineer types: vpn-id = 100 and name = ZTP-VPN CFS auto-calculates: import_rt = 65000:100 (65000 = your AS, 100 = vpn-id) export_rt = 65000:100 vrf_name = VRF-ZTP-VPN (VRF- prefix + vpn name) Engineer never types RT values or VRF name manually.

## 3.1 Package Creation Command

cd /nso/run/packages ncs-make-package --service-skeleton python l3vpn-cfs # Note: NSO converts dashes to underscores in folder names # Package folder: l3vpn-cfs/ # Python folder: l3vpn-cfs/python/l3vpn_cfs/ <-- underscore

## 3.2 YANG Model

File location: packages/l3vpn-cfs/src/yang/l3vpn-cfs.yang

module l3vpn-cfs { namespace "http://example.com/l3vpn-cfs"; prefix l3vpn-cfs; import ietf-inet-types { prefix inet; } import tailf-common { prefix tailf; } import tailf-ncs { prefix ncs; } description "L3VPN Customer Facing Service"; revision 2016-01-01 { description "Initial revision."; } list l3vpn-cfs { description "L3VPN CFS - takes simple inputs only"; key name; leaf name { tailf:info "Unique VPN service name"; tailf:cli-allow-range; type string; } uses ncs:service-data; ncs:servicepoint l3vpn-cfs-servicepoint; leaf vpn-id { type uint32; mandatory true; } leaf pe-device { type leafref { path "/ncs:devices/ncs:device/ncs:name"; } mandatory true; } } }

### 3.2.1 CFS YANG vs RFS YANG — Key Difference

| CFS YANG (l3vpn-cfs) | RFS YANG (vrf_creation_ios_xr) |
| --- | --- |
| leaf name — VPN name (key) | leaf name — service instance name (key) |
| leaf vpn-id — single number | leaf-list device — multiple devices |
| leaf pe-device — single device | leaf vrf — VRF name |
| NO import_rt or export_rt | leaf import_rt — mandatory |
| NO vrf name | leaf export_rt — mandatory |
| Simple: 3 inputs only | Technical: all details needed |

### 3.2.2 Why leaf pe-device instead of leaf-list device

The CFS uses a single leaf pe-device (not a leaf-list) because:

- CFS takes ONE simple order per VPN site from the engineer

- leaf-list allows multiple devices — used in RFS where config is pushed to many devices

- CFS keeps the interface simple — engineer types one device name

- If multiple sites are needed, create multiple CFS instances or extend the YANG model later

## 3.3 Python Service Code

File location: packages/l3vpn-cfs/python/l3vpn_cfs/main.py

Note: NSO converts dashes to underscores — folder is l3vpn_cfs not l3vpn-cfs

# -*- mode: python; python-indent: 4 -*- import ncs # Load the NSO Python library from ncs.application import Service # Import the Service base class class ServiceCallbacks(Service): # Our callback class inherits from Service @Service.create # Decorator: run this when service is CREATED def cb_create(self, tctx, root, service, proplist): # # Parameters: # self - the class itself # tctx - transaction context (info about current transaction) # root - the entire NSO database (CDB) - access everything # service - YOUR service instance (what engineer typed) # proplist - temporary storage between callbacks # self.log.info('CFS create(service=', service._path, ')') # ^ Write to NSO log for debugging. Shows service path when triggered. # ── STEP 1: Read inputs from engineer ────────────────────────── vpn_name = service.name # e.g. 'ZTP-VPN' vpn_id = service.vpn_id # e.g. 100 pe_device = service.pe_device # e.g. 'core-rtr01' # # Note: YANG uses dashes (vpn-id) but Python uses underscores (vpn_id) # NSO converts automatically. service.vpn_id reads leaf 'vpn-id' in YANG. # ── STEP 2: Internal mapping — auto calculate all technical values ── import_rt = f"65000:{vpn_id}" # 65000:100 (AS:vpn-id) export_rt = f"65000:{vpn_id}" # 65000:100 vrf_name = f"VRF-{vpn_name}" # VRF-ZTP-VPN # # f"65000:{vpn_id}" is a Python f-string. # It inserts the variable value inside {}. # If vpn_id = 100, then f"65000:{vpn_id}" becomes "65000:100" self.log.info(f'Calculated: vrf={vrf_name} import_rt={import_rt} export_rt={export_rt}') # ^ Logs the calculated values for debugging # ── STEP 3: Call the VRF RFS service ────────────────────────── vrf_instance = root.vrf_creation_ios_xr.create( f"{vpn_name}-vrf" # Instance name: e.g. 'ZTP-VPN-vrf' ) # # root = entire NSO CDB database # .vrf_creation_ios_xr = your RFS package/list name # .create('ZTP-VPN-vrf') = create a new RFS instance with this key # Fill in the RFS inputs with auto-calculated values vrf_instance.device.create(pe_device) # which router to configure vrf_instance.vrf = vrf_name # VRF-ZTP-VPN vrf_instance.import_rt = import_rt # 65000:100 vrf_instance.export_rt = export_rt # 65000:100 self.log.info('CFS successfully created VRF RFS instance') class Main(ncs.application.Application): # Entry point NSO starts on package load def setup(self): self.log.info('Main RUNNING') self.register_service('l3vpn-cfs-servicepoint', ServiceCallbacks) # ^ Must match ncs:servicepoint in YANG exactly # ^ Class with our logic def teardown(self): # Called when package is unloaded self.log.info('Main FINISHED')

## 3.4 Python Code — Section by Section

### 3.4.1 The Boilerplate (Never Changes)

This section is identical in EVERY NSO Python service you will ever write:

import ncs from ncs.application import Service class ServiceCallbacks(Service): @Service.create def cb_create(self, tctx, root, service, proplist):

Key Point Only the LOGIC inside cb_create changes for each service. The imports, class definition, decorator, and function signature are always the same. Think of it as a form — the form never changes, only what you fill in changes.

### 3.4.2 The cb_create Parameters Explained

| Parameter | What It Contains |
| --- | --- |
| self | Reference to the class — standard Python OOP |
| tctx | Transaction context — info about who triggered the service and when |
| root | The entire NSO CDB — you can read/write any data in NSO through root |
| service | Your service instance — contains what the engineer typed (service.name, service.vpn_id, etc.) |
| proplist | Temporary property list — used to store data between pre/post modification callbacks |

### 3.4.3 YANG Leaf to Python Attribute Mapping

| YANG Leaf Name | Python Access |
| --- | --- |
| leaf name | service.name |
| leaf vpn-id | service.vpn_id (dash becomes underscore) |
| leaf pe-device | service.pe_device (dash becomes underscore) |
| leaf import_rt | service.import_rt (underscore stays) |
| leaf-list device | service.device (iterable list) |

### 3.4.4 Step 2 — Internal Mapping Logic

This is where the CFS intelligence lives. The engineer types vpn-id = 100 and everything else is auto-calculated:

# Input from engineer: vpn_name = 'ZTP-VPN' vpn_id = 100 # Auto-calculated by CFS: import_rt = f"65000:{vpn_id}" --> '65000:100' export_rt = f"65000:{vpn_id}" --> '65000:100' vrf_name = f"VRF-{vpn_name}" --> 'VRF-ZTP-VPN'

### 3.4.5 Step 3 — Calling the RFS

The CFS creates an instance of the VRF RFS service programmatically. This is the same as the engineer typing the RFS commands manually but done automatically by code:

# What CFS does in Python: vrf_instance = root.vrf_creation_ios_xr.create('ZTP-VPN-vrf') vrf_instance.device.create('core-rtr01') vrf_instance.vrf = 'VRF-ZTP-VPN' vrf_instance.import_rt = '65000:100' vrf_instance.export_rt = '65000:100' # Equivalent to engineer manually typing in NSO CLI: # vrf_creation_ios_xr ZTP-VPN-vrf # device [ core-rtr01 ] # vrf VRF-ZTP-VPN # import_rt 65000:100 # export_rt 65000:100

# 4. How to Use the L3VPN CFS

## 4.1 Compile and Load

# Step 1: Compile the YANG model cd /nso/run/packages/l3vpn-cfs/src make # Step 2: Load into NSO ncs_cli -C -u admin packages reload # Expected output: reload-result { package l3vpn-cfs result true } reload-result { package vrf_creation_ios_xr result true }

## 4.2 Create the CFS Service — NSO CLI

# Enter config mode ncs_cli -C -u admin config # Create CFS with only 3 inputs l3vpn-cfs ZTP-VPN vpn-id 100 pe-device core-rtr01 # Check what will happen BEFORE committing (dry run) commit dry-run # Check native device commands that will be sent commit dry-run outformat native # Commit and push to router commit

## 4.3 What Happens Step by Step

1. Engineer types: l3vpn-cfs ZTP-VPN vpn-id 100 pe-device core-rtr01 ¦ v 2. NSO triggers cb_create() in Python ¦ v 3. Python reads: vpn_name = 'ZTP-VPN' vpn_id = 100 pe_device = 'core-rtr01' ¦ v 4. Python calculates: import_rt = '65000:100' export_rt = '65000:100' vrf_name = 'VRF-ZTP-VPN' ¦ v 5. Python creates RFS: vrf_creation_ios_xr ZTP-VPN-vrf device [ core-rtr01 ] vrf VRF-ZTP-VPN import_rt 65000:100 export_rt 65000:100 ¦ v 6. RFS template maps values to IOS-XR config ¦ v 7. Config pushed to router core-rtr01: vrf VRF-ZTP-VPN address-family ipv4 unicast import route-target 65000:100 export route-target 65000:100

## 4.4 Verify the Service

# Check CFS service instance show running-config l3vpn-cfs # Check RFS service instance created by CFS show running-config vrf_creation_ios_xr # Check actual device configuration devices device core-rtr01 live-status exec any "show run vrf" # Check sync status devices device core-rtr01 check-sync # Re-deploy if needed l3vpn-cfs ZTP-VPN re-deploy

## 4.5 Delete the Service

# Delete CFS instance (automatically deletes RFS instance too) config no l3vpn-cfs ZTP-VPN commit # NSO automatically rolls back: # - Removes l3vpn-cfs ZTP-VPN # - Removes vrf_creation_ios_xr ZTP-VPN-vrf # - Removes VRF config from router

# 5. RESTCONF API Reference

## 5.1 VRF RFS — Direct API Calls

### Create VRF Service

curl --location --request POST \ 'http://NSO_IP:8080/restconf/data/vrf_creation_ios_xr:vrf_creation_ios_xr' \ --header 'Content-Type: application/yang-data+json' \ --header 'Accept: application/yang-data+json' \ --user developer:C1sco12345 \ --data '{ "vrf_creation_ios_xr:vrf_creation_ios_xr": { "name": "MY-VRF", "device": ["core-rtr01"], "vrf": "BLUE-VRF", "import_rt": "65000:100", "export_rt": "65000:100" } }'

### Read VRF Service

curl --location --request GET \ 'http://NSO_IP:8080/restconf/data/vrf_creation_ios_xr:vrf_creation_ios_xr' \ --header 'Accept: application/yang-data+json' \ --user developer:C1sco12345

### Delete VRF Service

curl --location --request DELETE \ 'http://NSO_IP:8080/restconf/data/vrf_creation_ios_xr:vrf_creation_ios_xr=MY-VRF' \ --header 'Accept: application/yang-data+json' \ --user developer:C1sco12345

## 5.2 L3VPN CFS — API Calls

### Create CFS Service (simple 3-field input)

curl --location --request POST \ 'http://NSO_IP:8080/restconf/data/l3vpn-cfs:l3vpn-cfs' \ --header 'Content-Type: application/yang-data+json' \ --header 'Accept: application/yang-data+json' \ --user developer:C1sco12345 \ --data '{ "l3vpn-cfs:l3vpn-cfs": { "name": "ZTP-VPN", "vpn-id": 100, "pe-device": "core-rtr01" } }'

## 5.3 Device Sync API Calls

| Action | API Call |
| --- | --- |
| Sync FROM device (import device config to NSO) | POST /devices/device=core-rtr01/sync-from |
| Sync TO device (push NSO config to device) | POST /devices/device=core-rtr01/sync-to |
| Check sync status | POST /devices/device=core-rtr01/check-sync |
| Compare config | POST /devices/device=core-rtr01/compare-config |

# 6. Troubleshooting

## 6.1 Common Errors and Fixes

### Error: 'List' object has no attribute 'vrf_creation_ios_xr'

Problem Python is trying to navigate to the RFS service using wrong path.

# WRONG: vrf_instance = root.vrf_creation_ios_xr.vrf_creation_ios_xr.create(...) # CORRECT: vrf_instance = root.vrf_creation_ios_xr.create(...)

### Error: Package reload result false

# Check what went wrong: show packages package l3vpn-cfs oper-status # Check Python syntax: python3 -c "import ast; ast.parse(open('main.py').read())" # Check YANG compile: cd src && make

### Error: NSO out of sync with device

# Problem: Config was changed directly on router bypassing NSO # Solution: Sync from device curl -X POST \ 'http://NSO_IP:8080/restconf/data/devices/device=core-rtr01/sync-from' \ -u developer:C1sco12345 \ -H 'Accept: application/yang-data+json' # Or via CLI: devices device core-rtr01 sync-from

## 6.2 Useful Debug Commands

# Check package status show packages package oper-status # Check service instances show running-config l3vpn-cfs show running-config vrf_creation_ios_xr # Dry run before committing commit dry-run commit dry-run outformat native # Check NSO logs tail -f /nso/run/logs/ncs-python-vm.log # Re-deploy a service l3vpn-cfs ZTP-VPN re-deploy # Check what service owns which device config l3vpn-cfs ZTP-VPN get-modifications

# 7. Quick Reference

## 7.1 Python Service Template (Copy-Paste Starting Point)

Every new NSO Python service starts with this structure. Only the logic inside cb_create changes.

# -*- mode: python; python-indent: 4 -*- import ncs # ALWAYS SAME from ncs.application import Service # ALWAYS SAME class ServiceCallbacks(Service): # ALWAYS SAME @Service.create # ALWAYS SAME def cb_create(self, tctx, root, service, proplist): # ALWAYS SAME # ── YOUR LOGIC GOES HERE ────────────────────────────────── # 1. Read inputs (change leaf names to match your YANG) name = service.name vpn_id = service.vpn_id # YANG 'vpn-id' -> Python 'vpn_id' device = service.pe_device # YANG 'pe-device' -> Python 'pe_device' # 2. Calculate values internally import_rt = f"65000:{vpn_id}" vrf_name = f"VRF-{name}" # 3. Push config (either call RFS or configure device directly) rfs = root.your_rfs_package.create(f"{name}-instance") rfs.device.create(device) rfs.vrf = vrf_name # ───────────────────────────────────────────────────────── class Main(ncs.application.Application): # ALWAYS SAME def setup(self): # ALWAYS SAME self.log.info('Main RUNNING') # ALWAYS SAME self.register_service( # ALWAYS SAME 'YOUR-servicepoint-NAME', # CHANGE: must match YANG servicepoint ServiceCallbacks # ALWAYS SAME ) def teardown(self): # ALWAYS SAME self.log.info('Main FINISHED') # ALWAYS SAME

## 7.2 YANG Leaf Types Quick Reference

| YANG Type | Use For |
| --- | --- |
| type string | Any text value — VRF name, description, etc. |
| type string { length '1..32'; } | Text with length validation |
| type uint32 | Positive whole numbers — VPN ID, VLAN ID, etc. |
| type inet:ipv4-address | IPv4 address with format validation |
| type leafref { path '...' } | Reference to another list — validates value exists |
| leaf-list device | Multiple values of the same type — list of device names |
| mandatory true; | Field is required — cannot be left empty |

## 7.3 NSO Package Build Steps — Every Time

# 1. Edit YANG model nano packages/PACKAGE_NAME/src/yang/PACKAGE_NAME.yang # 2. Edit Python code nano packages/PACKAGE_NAME/python/PACKAGE_NAME/main.py # 3. Compile YANG cd packages/PACKAGE_NAME/src && make # 4. Reload into NSO ncs_cli -C -u admin packages reload # 5. Test config PACKAGE_NAME INSTANCE_NAME ... commit dry-run commit

## 7.4 File Locations Summary

| File | Location |
| --- | --- |
| VRF YANG model | packages/vrf_creation_ios_xr/src/yang/vrf_creation_ios_xr.yang |
| VRF XML template | packages/vrf_creation_ios_xr/templates/vrf_creation_ios_xr-template.xml |
| CFS YANG model | packages/l3vpn-cfs/src/yang/l3vpn-cfs.yang |
| CFS Python code | packages/l3vpn-cfs/python/l3vpn_cfs/main.py |
| NSO Python logs | /nso/run/logs/ncs-python-vm.log |
| NSO main log | /nso/run/logs/ncs.log |

— End of Document —